PIC16F72 Instruction-Set Analysis Study

1. Overview

- Our project is a software simulator for the PIC16F72 microcontroller.

- We analyzed the instruction set to understand how the chip executes its 35 commands.

- Every instruction is fixed at 14 bits and stored in the 2K × 14-bit program memory.

2. How the 14-Bit Instruction is Split Up

Every 14-bit instruction contains the operation type along with its target variables:

- The d bit (Destination):

  - d = 0 -> Saves the result in the Working Register (W).

  - d = 1 ->Saves the result back into the memory variable (f).

- The b bits (Bit Picker): Selects a single bit position from 0 to 7.

- The k bits (Literal Value): Holds a fixed number or a target jump address.

3. The 4 Instruction Types

- 1. Byte Operations (Math & Data Movement):

  - ADDWF count, 0 → Adds count to W and stores the answer in W.

  - MOVWF count → Copies the value from W into count.

  - CLRF count → Resets count to 0.

- 2. Bit Operations (Single Pin / Flag Control):

  - BSF PORTB, 0 → Sets Pin 0 to HIGH (turns an LED ON).

  - BCF PORTB, 0 → Sets Pin 0 to LOW (turns an LED OFF).

  - BTFSC PORTA, 0 → Checks button on Pin 0; skips the next instruction if it is 0.

- 3. Literal Operations (Direct Numbers):

  - MOVLW 5 → Loads the number 5 directly into W.

  - ADDLW 10 → Adds 10 directly to W.

- 4. Control Operations (Flow Control):

  - GOTO main → Jumps directly to the label main.

  - CALL delay → Jumps to a function and saves the return address on the stack.

  - RETURN → Jumps back to where the program left off.

4. Instruction Timing & Cycles

- 1-Cycle Instructions: Standard operations like ADDWF or BSF execute in 1 instruction cycle (4 clock ticks).

- 2-Cycle Instructions: Jump and branch instructions (GOTO, CALL, or successful skips) take 2 instruction cycles because the pipeline reloads.

- Timer0 Interaction: Timer0 adds +1 for single-cycle instructions and +2 for two-cycle instructions.

5. Simulator Modules Identified

| Simulator Module | What It Does |
| --- | --- |
| Execution Core | Runs arithmetic/logic operations, updates W, and writes to memory. |
| Control Logic | Updates the Program Counter (PC) and handles the 8-level return stack. |
| Status Flags | Updates the Zero (Z) and Carry (C) flags in the STATUS register after math operations. |
| Timing Tracker | Counts 1-cycle and 2-cycle execution to advance Timer0 accurately. |

6. Conclusion

This study breaks down the behavior, categories, and cycle timing of the 35 instructions. It serves as a clear reference to guide the implementation of the CPU execution logic in our simulator.